# OPTIMAL DESIGN OF RENEWEBLE ENERGY SYSTEM USING GENETIC ALGORITHM:

# CASE STUDY IN PARANGTRITIS

Bagian ini sengaja dikosongkan

SRI UTAMI

Departement of Energy Convertion Engineering, Bandung State Polytechnic

Email : utamiwu@yahoo.com, sri.utami@polban.ac.id

## Abstract

Tourism has a massive contribution to regional development also share environmental issues. Reducing reliances on fossil fuel yet still adequating energy consumption cause development of reneweble energy in crucial position for tourism desicition. Optimal configuration of reneweble energy system energy are planned using Genetic Algorithm in this work. This research conducted to optimize reneweble  energy system in Parangtritis, Kretek, Bantul Central Java. The system consists of solar cells and wind turbines, the storages used are batteries and fuel cells. The algorithm minimize the objective function consisting of investment, replacement as well as operating and maintenance costs. Reability evaluation of the system given by Equivalent Loss Factor (ELF). This index inform insufficient energy in the systems. The simulation showed, optimum size of the system  achieved using size of PV, battery, wind turbine, electrolizer, hidrogen tank and fuel cell are  3, 48, 36, 3, 2, 8 respectively. The ELF used in this simulation is 0.01.

Keywords: fossil fuel,  reneweble energy, tourism, reability, Equivalent Lost Factor

## 1. introduction

Tourism is one of important economic driver of a region. At the same time, it still very depends on fosil energy sources, that are very vulnerable to fuel availability as well as the rise and fall of oil costs, to meet energy needs. Integration of reneweble energy, using appropriate combination,  is able to compensate the weakness of one source using advantages of the other source.

Variation of several combinations, both renewable energy sources and storage media, and system cost optimization have been done by several researchers. Yang et al (2008) proposed optimization of hybrid system of wind turbine, solar cell and battery banks using loss of power supply probability (LPSP) methods. Hakimi and Tafreshi (2009) performed optimization in Kahnouj in the southeastern region of Iran using particle swarm. In this research, Hakimi and Tafreshi (2009) assumed stand-alone system consisting fuel cells, some wind units, some electrolyzers, a reformer, an anaerobic reactor and some hydrogen tanks. Optimal cost for stand-alone hybrid reneweble energy system also has been found by Tolooi et al (2014) using evolutionary algorithm. This algorithm based on collective intelligence (gravitational search algorithm). Furthermore, Tolooi et al (2014)  also found optimal value for hydrid system using particle swarm optimization as comparation.

The method proposed in this study is the genetic algorithm. This algorithm is widely used because of its ability to find an optimal solution to an optimization problem. In this study, genetic algorithm is used to optimize the design of renewable energy systems for the area Parangtritis consisting of solar cells, wind turbines, batteries and fuel cells. Design of optimal system minimizes the net present cost (NPC), including the cost of the investment (capital cost), the cost of the replacement (replacement cost), and operating and maintenance costs (operation and maintenance cost).

## 2. hyrid system

The hybrid system combines two or more sources of energy, from renewable or conventional energy sources, with storage media. This hybrid system design can be different following the renewable energy sources available locally to meet the load requirements. Parangtritis located in Kretek, Bantul, Central Java, has a high potential of hybrid renewable energy system considering the geographical location as well as the intensity of the sun and wind velocity.

Wind Turbine Angin

Solar cell

Battery

Fuel Cell

AC Load

DC bus

AC bus

Figure 1. Configuration of reneweble energy system

Integration of several sources and different media used in this work as ilustrated in Figure  1.  Reneweble energy sources are wind turbine and solar cell. As a storage media, battery and fuel cell are used.

### 2.1 Reneweble Energy Componenets

### To predict the performance of the system, first step is how to model each of individual components of the system. After modeling, every component can be combined for further evaluation to meet the load requirements.

### 2.1.1 Photovoltaic

Solar panel power output is influenced by insulation condition. Suryoatmojo (2010) and Penangsang et al (2013) used Equation 1 for the modeling of solar cells

P PV(t) ApN PVE(t)                  ...................................................................................(1)

For  efisiensi energy conversion (%), AP is the area of the solar panel (m2), NPV is the number of photovoltaic panels, E (t) is the data solar insulation per hour (W / m2).

2.1.2 Wind Turbine

Determination of wind turbine modelling is crucial for the output power. Yusuf (2012) used mathematic model of Equation 2

=

Pwg,t      0vt<vcutin,   vt>vcutoutPmaxvt-vcutinvrated-vcutinvcutin≤vt≤vratedpmax+pfurl-pmaxvcutout-vrated.(vt-vrated)vrated<vt≤vcutout    ..................................(2)

Where vt is the wind speed in one step a time, vcutin is the value when the cut-in wind speed, vcutout is the value of the current wind speed in cut out and vrated is the value of wind speed (rate) of the wind turbine (m/s). While pmax is the maximum output power and pfurl is the power output at a speed of cut-outs (kW) calculated after passing through the inverter.

2.2  Storage Components

2.2.1 Battery

There are two conditions need to be considered related to charging and discharge condition for the battery at the time t. While battery charging, Suryoatmojo (2010) used Equation 3

Cbat(t) =Cbat(t-1)+(Pren(t)-P load(t))........................................................................................(3)

And for discharging Equation 4 is used,

Cbat(t) =Cbat(t-1) - (Pload(t)-Pren(t)).........................................................................................(4)

where Cbat(t) is the capacity of the battery during the time t, Cbat(t-1) is the capacity of the battery at previous time.

2.2.2 Fuel Cell

The fuel cell system consists of electrolizer, hydrogen tank and fuel cell. A mathematical formula is as follows:

2.2.2.1 Electrolizer

The power generated from the electrolysis process obtained using Equation 5 (Yusuf, 2012)

Pel,t=(Pren,t - Pload,t )..............................................................................................(5)

Pel,t is the electrical power delivered to the electrolyzer from renewable system

2.2.2.2 Hydrogen Tank

Energy flow into the hydrogen tank  can be explained by the Formula 6 (Yusuf, 2012)

Ctank,t=Ctank,t-1+Pel,t .............................................................................................(6)

Ctanks, t and C tanks, (t-1) are the total amount of energy in the hydrogen tank when t and the previous time (t-1) respectively.

2.2.2.3 Fuel Cell

Output from the fuel cell is proportional to the input power of hydrogen calculated by Equation 7 (Yusuf, 2012)

Pfc,t=Pload,t -Pren,t ............................................................................................(7)

2.3 Genetic Algorithm

The concept of genetic algorithm is different from the traditional search and optimization methods in the engineering problems. In this algorithm, basic principle of genetic processes in biology is imitated in search process. This technique was introduced to search the optimal solution based on natural selection (Suryoatmojo,2010).

In the genetic algorithm, like other algorithms in search optimization, requires defining optimization variables and function costs. Like another algorithm, the value of convergence will be tested.

Main components using in genetic algorithm is as follows (Bramantyo, 2006):

1. Coding Scheme
2. Population
3. Natural selection 
4. Selection
5. Mating
6. Mutation
7. The next generation

Cycles of genetic algorithm is depicted in figure 2.

End

Calculate the cost for each chromosome

Select parent pair

(Select Mate)

Mating

Mutation

Check convergence convergence

Define the cost function, the cost, the variables,
select the GA parameters

Generate initial population

Read chromosome

Figure 2. Flowchart of genetic algorithm

2.4 Problem Formulations

2.4.1 Objective Function (Economic Modelling Based on Net  Present Cost (NPC))

Cost component was calculated using the Net Present Cost (NPC) consisting of  cost of the investment (capital costreplacement cost, and operating and maintenance costs . This cost will be calculated by the value of money at this time.

This method can be described by Equation 8:

NPC=CCi+RCi∙Ki+MCi∙PWA(ir,R)..............................................................................(8)

Where CCi is the investment cost ($ / unit). RC is the replacement cost ($ / unit). While MCi  is operating and maintenance costs ($/unit-year). Replacement cost RCi will be multiplied by Ki. A conversion factor value for money at a certain moment in the future to the current time because the calculation of the cost calculated by the value of money at this time. While the operating and maintenance costs will be multiplied by PWA (annual payment present worth) that is used to convert annual cost of operating and maintaining backwards today.

2.4.2 Constrain

Some restriction were used to optimize objective function in renewable generation systems is as follows:

(1) The balance of power system

(Pt,t≥Pload,t-Qt) .....................................................................................................(10)

(2) Index of reliability system

E[ELF]≤ELFmax .......................................................................................................(11)

(3) Number of units and equipment

Nwt, Npv, Nbat, Nel, Ntank, Nfc ≥0........................................................................................(12)

(4) Boundary of batteries

Cbatmin≤Cbat(t) ≤Cbatmax.............................................................................................(13)

(5) Limitation of Hidrogen Tank

Ctankmax≤Ctank(t) ≤Ctankmax.........................................................................................(14)

2.4.3 Equivalent Loss Factor (ELF)

Equivalent Loss Factor will compare the difference between the energy produced to the load requirements in one year. Bashir and Garcia and Weisser (2006) calculated ELF using equation (15)

ELF = 1H  i=1HQ (i)D(i) .....................................................................................................(15)

H is the total number of time steps (8760), Q(i) is the total load that can not be met and D (i) the total load demand per time step.

2.5 Operational Strategy

Operational strategy used in this reneweble energy system is depicted as follow:

1. When the total load demand load is smaller than the output produced renewable generation systems, the remaining energy will be stored into the battery

2. When the battery capacity is full (and there is still residual energy) charging will be continued to the fuel cell

3. When the fuel cell has the maximum capacity, the excess energy will be discharged into dump load

4. When the load demand is greater than the power that can be supplied by renewable generation, the shortage will be supplied by the battery

5. When the load demand remains greater, despite being supplied by the battery, the energy shortage will be fulfilled by fuel cell

3. IMPLEMENTATION

The proposed methode implemented using flowchart depicted in Figure (3)

Figure 3. Implementation of Genetic Algorithm

3.1 Simulation

Parangtritis was  used for the case study in this study to design the hybrid system simulation. Daily load demand characteristic in Parangtritis is shown in Figure 4.This simulation used data per hour for one day and repeated for one year or 8760 hours. The time period of this project was 20 years. The specification of PV panel, wind turbine, battery and fuel cell used in this study can be seen in Table 1, 2, 3 and 4. The parameters of GA used in this study are shown in Table 5.

Figure 4. Daily load profile

The solar panels used are the solar cell modules of Canadian Solar CS6P-240p (K.M. Iromi ,2013).

Table 1. Specifications solar cell module Canadian Solar CS6P-240p (at standard test conditions Irradiance 1000W / m2, AM1.5 spectrum and solar cell temperature 25 ° C) (K. M. Iromi, 2013)

| Nominal Maximum Power | Pmpp | 240 W |
| --- | --- | --- |
| Optimum operating voltage | Vmpp | 29.9 V |
| Optimum operating current | Impp | 8.03 A |
| Open circuit voltage | Voc | 37.0 V |
| Short circuit current | Isc | 8.59 A |
| Module efficiency | η | 14.92 % |
| Temperature coefficient at Pmpp |  | -0.43% °C-1 |
| Nominal Operating Cell Temperature (NOCT) |  | 45 ± 2°C |
| Capital Cost | CC | 7000 ($/unit) |
| Replacement Cost | RC | 6000($/unit) |
| Maintenance Cost | MC | 20($/unit) |

BWC Excel-R / 48 artificial Bergey Wind Power's used in this study. The specifications of this wind turbine is shown in Table 2(A Navaeefard et al, 2010),(M. Bashir and J. Sadeh ,2012), (Nowdeh et al, 2010).

Table 2 Specifications of Wind Turbines

| Cut-in Speed | vcutin | 3(m/s) |
| --- | --- | --- |
| Cut-out Speed | vcutout | 25(m/s) |
| Rated Speed | vrated | 15(m/s) |
| Maximum Output Power | pmax | 8,1(kW) |
| Maximum Output Power Cut-out | pfurl | 5,8(kW) |
| Capital Cost | CC | 16.400($/Unit) |
| Replacement Cost | RC | 13.000($/Unit) |
| Maintenance Cost | MC | 400($/Unit-year) |

For this study the battery used is Surrette-6CS25P the assumed life of the battery is 10 years old and the depth of discharge is about 80%.

Table 3. Specifications Battery Storage (M. Bashir, 2012)

| Battery Capacity |  | 6,94 (kWh) |
| --- | --- | --- |
| Charge Efficiency | ηcharge | 0,9 |
| Discharge Efficiency | ηdischarge | 0,9 |
| Time periode |  | 10 (tahun) |
| DOD |  | 0,8 |
| Capital Cost | CC | 1.250($/Unit) |
| Replacement Cost | RC | 1.200($/Unit) |
| Maintenance Cost | MC | 20($/Unit-year) |

Table 4. Fuel cell Spesifications (http://hypertextbook.com)

| Capacity |  | 0,5kWh |
| --- | --- | --- |
| Efficiency | ηfc | 0,7 |
| Time Period |  | 20(tahun) |
| Capital Cost | CC | 3.000($/Unit) |
| Replacement Cost | RC | 2.500($/Unit) |
| Maintenance Cost | MC | 175($/Unit-year) |

The algorithm parameters used in Table 5 below.

Table 5. Genetic Algorithms Parameters

| Population | 32 |
| --- | --- |
| Mutation rate | 0.02 |
| Crossover | One point crossover |
| Iteration | 120 |

4. RESULT

The total cost of the system can be specified for each element in renewable generation systems as shown in Figure 5. The investment costs for the system built is $ 711,120, while the cost of battery replacement, elektrolizer, hydrogen tank and a fuel cell for $ 29,484. In this study assumed replacement battery as much as one but not the other component replacements performed over the life of the project. The cost for operating and maintenance costs of the system is $ 57,970

Figure 5.

In this simulation crossover rate used is one point crossover, while the probabilty mutation (mutation rate) used was 0.2. When the maximum number of iterations reached, an optimal configuration is obtained. In the last iteration can be concluded cost required for these renewable energy systems is $ 798,570 for system configuration as shown in Table 6.

Table 6.

| No | Components | Size |
| --- | --- | --- |
| 1 | Solar cell | 3 |
| 2 | Batteries | 48 |
| 3 | Wind turbine | 36 |
| 4 | Electrolizer | 3 |
| 5 | Tangki hidrogen | 2 |
| 6 | Fuel cell | 8 |

The cost for each of component as in Figure 6.

Figure 6. Figure 13 Cost for each system component

5. REFERENCES

A. Navaeefard, S.M.M. Tafreshi, M. Barzegari and A.J. Shahrood (2010)"Optimal Sizing of Distributed Energy Resources in Microgrid Considering Wind Energy Uncertainty with Respect to Reliability"

Garcia,R. S dan Weisser, D (2006) "A Wind-Diesel System with Hydrogen Storage Joint Optimization of Design and Dispatch" Reneweble energy, Vol. 31, No.14, pp.2296-2320

Hakimi, S.M and S.M. Moghaddas-Tafreshi (2009). Optimal Sizing of a Stand-alone Hybrid Power System via Particle Swarm Optimization for Kahnouj area in south-east of Iran. Renewable Energy 34: 1855–1862. 2009

K. M. Iromi Udumbara Ranaweera (2013). Techno-Economic Optimum Sizing of Hybrid Renewable Energy System -Rural Electrification In Srilanka-, Master Thesis, University of Agder, Grimstad, Norway, June

M. Bashir dan J. Sadeh (2012)"Size Optimization of New Hybrid Stand-alone Renewable Energy System Considering a Reliability Index" IEEE

Nowdeh, S. A., Ghahnavieh, A. A., and Khanabdal, S (2010)" PV/FC/Wind Hybrid System Optimal Sizing Using PSO Modified Algorithm" Buletinul Institutului Politehnic Din IASI

Penangsang, O., Abdillah,M.,Wibowo,RS., Soeprijanto,A. (2013) " Optimal Design of Photovoltaic–Battery Systems Using Interval Type-2 Fuzzy Adaptive Genetic Algorithm

Suryoatmojo, H. (2010). Artificial Intelligence Based Optimal Configuration of Hybrid Power Generation System. Dissertation Doctor of Philosophy in Department of Computer Science and Electrical Engineering

Tolooi, Z., Zayandehroodi,H., and Khajehzadeh,A (2014). Novel Approach for Optimal Sizing of Stand-alone Hybrid Photovoltaic/Wind Systems Using Evolutionary Algorithms. ACSIJ Advances in Computer Science: an International Journal 3, Issue 6: 12

Yang, H., Zhou,W.,Lu,L., and Fang,Z (2008). Optimal Sizing Method for Stand-alone Hybrid Solar–wind System with LPSP Technology by using Genetic Algorithm. Solar Energy 82 (2008) 354–367

Yusuf, E. (2012). Optimasi Jumlah Unit Pembangkit pada Konfigurasi Sistem Hibrid Energi Terbarukan dengan Particle Swarm Optimization

http://hypertextbook.com/facts/2005/MichelleFung.shtml